Centronics port

ADRES &H37A:  (low nibble used)

Uitsturing van PortOut &H37A, 0 levert:		Uitsturing van PortOut &H37A, 15 levert:

Bit 0 (pin 1, strobe) : +5V			0V

Bit 1 (pin 14         ) : +5V			0V

Bit 2 (pin 16          ): 0 V			5V

Bit 3 (pin 17        ) : + 5V			0V

Data flow protocol:

1.- 	write data to port &H378

2.-	strobe bit 0 of port &H37A low and back high - pulse duration: minimum 1 microsecond

3.-	on the falling edge of the strobe signal the busy line should go high

4.-	the busy line should stay high for a maximum of 1ms and can go low again as soon as the interface is ready

5.-	When the interface is ready it strobes ACK low. And back high. Thus, on the rizing edge of ACK the busy line should go low.

ADRES &H379 - bit position d7,d6,d5,d4,d3 (only 5 bits used and connected on port)

Busy: 		bit 7	(pin 11)

Ack: 		bit 6	(pin 10)

Paperend: 	bit 5	(pin 12)

Select 		bit 4	(pin 13)

Error 		bit 3	(pin 15)

Reading of bit 7 is inverted on the LPT printer port: if the pin is electrical at +5V, it reads out %False.

If we want to make a I/O device to behave like a printer port, we need inverters on following pins/bit positions:

&H37A - bit 1 - pin 14

&H37A - bit 3 - pin 17

&H379 - bit 7 - pin 11 (DB25)